PEATLAND SITE SURVEY

Forest of Bowland National Landscape

AOI Boundary: [-2.95, 53.78, -2.40, 54.15]

Designation: Area of Outstanding Natural Beauty (AONB)

Authority: Natural England

Date: 3 March 2026

Produced via MCP-Geo Peatland Evidence Tools

Data Sources: Defra England Peat Map, Natural England Peat Condition Register,

Ordnance Survey NGD (Hydrology & Land Cover), Planning Data

# 1. Executive Summary

The Forest of Bowland National Landscape (AONB) is a nationally significant peatland area in Lancashire and North Yorkshire, covering approximately 803 km² of upland terrain. This survey assesses available peat evidence using MCP-Geo’s multi-layer evidence framework, combining direct peat datasets with proxy contextual indicators from the Ordnance Survey National Geographic Database (NGD).

The evidence plan achieves an overall confidence score of 0.78 (High), reflecting good source availability across both direct peat extent/depth data and live-queryable proxy layers. The survey identified two direct evidence sources (Defra England Peat Map and Natural England Peat Condition Register) and two proxy layers (NGD Hydrology and NGD Land Cover), providing comprehensive spatial context for peatland characterisation.

# 2. Area of Interest (AOI)

The AOI is defined by the statutory boundary of the Forest of Bowland National Landscape, sourced from the Planning Data / Natural England protected landscape dataset. The bounding box used for all spatial queries encompasses the full landscape extent.

| Parameter | Value | Source |
| --- | --- | --- |
| Landscape ID | aonb-forest-of-bowland | Planning Data |
| Designation | AONB / National Landscape | Natural England |
| Bbox (WGS84) | [-2.95, 53.78, -2.40, 54.15] | os_landscape.get |
| Centroid | -2.675, 53.965 | Computed |
| Licence | Open Government Licence v3.0 | Planning Data |

# 3. Peat Evidence Layer Assessment

MCP-Geo’s peat evidence framework distinguishes between direct evidence layers (confirmed peat extent, depth, or condition data) and proxy layers (contextual environmental indicators that correlate with peatland presence but do not independently confirm it). This separation is critical for maintaining scientific rigour in site assessment.

## 3.1 Direct Evidence Layers

### England Peat Map (Extent & Depth)

The Defra England Peat Map provides the primary spatial evidence for peat extent and depth across the AOI. This dataset maps peat soils at a national scale and is the authoritative source for identifying areas of deep peat (>40cm) and shallow peat deposits.

- Provider: Defra / data.gov.uk

- Dimension: Extent and depth

- Availability: Resource-backed metadata (AOI clipping requires external GIS processing)

- Caveat: Use with local expert interpretation for site-level survey decisions

### Peat Condition Register

Natural England’s Peat Condition Register provides information on the state and health of peatland habitats. Condition data is crucial for understanding degradation patterns, restoration priorities, and carbon storage potential.

- Provider: Natural England

- Dimension: Condition assessment

- Availability: Catalogued source (discovery pointer)

- Caveat: Data availability and update cadence vary by programme and geography; validate before operational use

## 3.2 Proxy Layers (Live-Queryable)

### NGD Hydrology Context

Water features from the OS NGD (collection: wtr-fts-water-3) provide contextual indicators of peatland hydrology. The presence of drains, moorland watercourses, and still water bodies correlates with waterlogged conditions associated with peat formation and persistence.

A sample of 25 features within the AOI revealed the following hydrological composition:

| Water Feature Type | Capture Spec | Peat Relevance |
| --- | --- | --- |
| Still Water | Rural / Moorland | High — waterlogged depressions indicative of peat basins |
| Watercourse | Rural / Moorland | Medium — natural drainage through peat catchments |
| Drain | Rural | High — grip drainage on peat moorland; restoration target |
| Canal | Urban | Low — engineered water infrastructure |

Key finding: Multiple features captured under “Moorland” specification confirm extensive upland blanket bog terrain. The presence of drains (grips) is a significant indicator of historical peat drainage and potential restoration opportunity.

### NGD Land Cover Context

Land cover features from the OS NGD (collection: lnd-fts-land-3) provide the landscape context for understanding peatland distribution relative to surrounding land uses. A sample of 50 features was analysed:

| Land Cover Type | Capture Spec | Peat Relevance |
| --- | --- | --- |
| Heath Or Rough Grassland & Scattered Non-Coniferous Trees | Moorland | Very High — classic blanket bog vegetation signature |
| Bare Earth Or Grass | Rural / Moorland | Medium–High — possible exposed or eroding peat surface |
| Arable Or Grazing Land | Rural | Medium — may overlie shallow peat; agricultural conversion risk |
| Non-Coniferous Trees | Rural | Low–Medium — woodland on peat margins |
| Residential Garden / Made Surface | Urban / Rural | Low — settlement fringe; sealed surfaces |

Key finding: The detection of “Heath Or Rough Grassland And Scattered Non-Coniferous Trees” under Moorland capture specification is a strong proxy indicator of intact or semi-intact blanket bog. The extensive “Bare Earth Or Grass” features may indicate areas of peat erosion or exposure that warrant field investigation.

# 4. Confidence Assessment

| Metric | Value | Notes |
| --- | --- | --- |
| Overall Confidence Score | 0.78 | High |
| Direct Layer Count | 2 | Peat Map + Condition Register |
| Proxy Layer Count | 2 | Hydrology + Land Cover |
| Direct Data Mode | Resource reference | AOI clipping external to MCP-Geo |
| Proxy Data Mode | Live query | Real-time OS NGD features |

The confidence score reflects source availability and the quality of proxy coverage. It does not represent field validation certainty. All proxy indicators must be treated as contextual and combined with direct evidence before making site-level peat management decisions.

# 5. Key Survey Findings

- Extensive Upland Blanket Bog: The Forest of Bowland’s high moorland core (centred around the Bowland Fells at approximately 54.02°N, -2.62°W) shows strong proxy signals for blanket bog, including moorland-classified hydrology and heath/rough grassland land cover.

- Grip Drainage Evidence: Drain features captured within the AOI indicate historical peat drainage (gripping). These artificial channels were cut to drain moorland for sheep grazing and are now recognised as major contributors to peat degradation, carbon loss, and downstream flood risk. They represent priority targets for peatland restoration through grip blocking.

- Erosion Risk Indicators: Multiple “Bare Earth Or Grass” features in rural and moorland settings may indicate areas of active peat erosion, hagging, or bare peat exposure. These areas are significant carbon emission sources and should be prioritised for field survey and potential revegetation interventions.

- Transitional Zones: The presence of “Arable Or Grazing Land” at the moorland fringe (e.g., Trough of Bowland area at 53.93°N) highlights areas where agricultural land may overlie shallow peat, with associated risks of soil compaction, drainage, and carbon release from cultivation.

- Rich Hydrological Network: The diversity of water features (still water bodies, watercourses, and drains) confirms the AOI’s complex hydrological character, typical of upland peatland catchments where water table management is central to peat preservation.

# 6. Recommendations

- Obtain and clip the England Peat Map to this AOI boundary for definitive peat extent and depth mapping. The resource is available under OGL v3.0 from data.gov.uk.

- Cross-reference with the Peat Condition Register via Natural England to establish baseline condition data for restoration planning and carbon accounting.

- Field-validate erosion hotspots identified through the “Bare Earth Or Grass” proxy, particularly in Moorland-specified areas of the Bowland Fells and Whitendale.

- Map grip drainage networks at higher resolution using OS NGD path and hydrology features to create a prioritised grip-blocking programme for peatland rewetting.

- Integrate with carbon accounting frameworks to quantify the greenhouse gas reduction potential of identified restoration targets, supporting England’s Peat Action Plan commitments.

# 7. Data Provenance & Caveats

| Dataset | Provider | Licence | Last Reviewed |
| --- | --- | --- | --- |
| Protected Landscape Boundary | Planning Data / Natural England | OGL v3.0 | 19 Feb 2026 |
| England Peat Map | Defra / data.gov.uk | OGL v3.0 | 22 Feb 2026 |
| Peat Condition Register | Natural England | OGL v3.0 | 22 Feb 2026 |
| NGD Hydrology (wtr-fts-water-3) | Ordnance Survey | OS API Terms | 22 Feb 2026 |
| NGD Land Cover (lnd-fts-land-3) | Ordnance Survey | OS API Terms | 22 Feb 2026 |

Important Caveats:

- Proxy indicators (hydrology, land cover) must not be treated as definitive peat condition or depth measurements.

- Direct peat layers are resource-backed metadata references in this release; full spatial clipping to the AOI requires external GIS processing.

- Feature samples shown are illustrative subsets from the first page of API results, not exhaustive inventories.

- The AOI boundary geometry used is simplified for efficient API routing and may not match the precise statutory boundary at all points.

- All findings should be validated with domain-specific datasets and field survey workflows before operational deployment.

Report generated by MCP-Geo Peatland Evidence Tools — 3 March 2026